Revisions to TASM not covered in the 2.8 documentation:

ENABLE RANGE CHECK FOR 8051 AJMP/ACALL.

The assembly control flag (-a)  may be used to enable this  optional

error  check.   To enable,  invoke TASM with '-a10'  on the  command

line.   This check will ensure that a branch off the current 2K byte

page will result in an error message.

WORD ADDRESSING

A  word (16  bit)  addressing mode was added to support those target

processors that  use word addresses instead of byte  addresses (e.g.

TMS320).

CONVERSION TO MICROSOFT C 6.0 AND TURBO C++

As of version 2.8.3 the TASM executable is built using TURBO C++.

Makefiles for both TURBO C and Microsoft C are provided, however.

Use:

MAKEFILE.TC             for TURBO C++

MAKEFILE.MSC            for Microsoft C 6.0

LOCAL LABELS

Labels that start with the '_'  character are considered to be local

to  a  given module and may be reused in subsequent  modules.    The

scope of a module is determined by a new directive (MODULE).

Here is an example:

.module xxx

lda regx

jne _skip

dec

_skip   rts

.module yyy

lda regy

jne _skip

dec

_skip   rts

In the above example,  the _skip label is reused without harm.  As a

default,   local  labels  are not shown in the label  table  listing

(resulting  from  the '-l'  command line  option).    See  following

section for options on the label table listing.

LONG FORM LABEL TABLE LISTING

Two options have been added (as suffixes) to the '-l' option which

causes a label table to be generated in the listing file.  They are

as follows:

Secondary option       Description

-------------------------------------------------

l                      Use long form listing

a                      Show all labels (including local labels)

The secondary flag should immediately follow the the '-l'.  For

example:

-l              to show non-local labels in the short form

-lal            to show all labels in the long form listing

-ll             to show non-local labels in the long form

ENHANCED 6800 FAMILY TABLE TO SUPPORT 68HC11

Use the '-x7' (or just 'x') command line option to enable the 68HC11

instructions.    TASM deviates from standard Motorola syntax for the

BSET,  BRSET,  BCLR,  and BRCLR instructions.   TASM requires commas

separating  all arguments.   Motorola assemblers use white space  to

separate the last one or two arguments for these instructions.  Here

are examples of each applicable instruction:

TASM                            MOTOROLA

----------------------          --------------------

BCLR    addr1,Y,bmsk            BCLR    addr1,Y bmsk

BCLR    addr1,X,bmsk            BCLR    addr1,X bmsk

BCLR    addr1  ,bmsk            BCLR    addr1   bmsk

BSET    addr1,Y,bmsk            BSET    addr1,Y bmsk

BSET    addr1,X,bmsk            BSET    addr1,X bmsk

BSET    addr1  ,bmsk            BSET    addr1   bmsk

BRCLR   addr1,Y,bmsk,lab1       BRCLR   addr1,Y bmsk lab1

BRCLR   addr1,X,bmsk,lab1       BRCLR   addr1,X bmsk lab1

BRCLR   addr1  ,bmsk,lab1       BRCLR   addr1   bmsk lab1

BRSET   addr1,Y,bmsk,lab1       BRSET   addr1,Y bmsk lab1

BRSET   addr1,X,bmsk,lab1       BRSET   addr1,X bmsk lab1

BRSET   addr1  ,bmsk,lab1       BRSET   addr1   bmsk lab1